PROGRAM : SKETCH.IO

----------------------------------

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// =====================================

// PIN DEFINITIONS

// =====================================

const int SIGNAL_LED = 2;      // Yellow LED - GO signal

const int WIN_LED = 3;         // Green LED - Result

const int BUTTON_PIN = 4;      // Push button

const int RED_LED = LED_BUILTIN;

// Performance bar LEDs

const int BAR_LED_1 = 5;

const int BAR_LED_2 = 6;

const int BAR_LED_3 = 7;

const int BAR_LED_4 = 8;

const int BAR_LED_5 = 9;

// =====================================

// LCD

// =====================================

LiquidCrystal_I2C lcd(0x27, 16, 2);

// =====================================

// GAME STATES

// =====================================

enum GameState {

WAITING,

COUNTDOWN,

GO,

RESULT,

FALSE_START

};

GameState gameState = WAITING;

// =====================================

// VARIABLES

// =====================================

unsigned long startTime = 0;

unsigned long reactionTime = 0;

bool lastButtonState = HIGH;

// =====================================

// BUTTON FUNCTION

// =====================================

bool buttonPressed() {

bool currentState = digitalRead(BUTTON_PIN);

if (lastButtonState == HIGH && currentState == LOW) {

delay(20);

lastButtonState = currentState;

return true;

}

lastButtonState = currentState;

return false;

}

// =====================================

// READY SCREEN

// =====================================

void showReady() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Reaction Game!");

lcd.setCursor(0, 1);

lcd.print("Press Button");

}

// =====================================

// COUNTDOWN

// =====================================

void showCountdown(int number) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Get Ready...");

lcd.setCursor(7, 1);

lcd.print(number);

}

// =====================================

// GO SCREEN

// =====================================

void showGo() {

lcd.clear();

lcd.setCursor(5, 0);

lcd.print("GO!!!");

lcd.setCursor(0, 1);

lcd.print("PRESS NOW!");

}

// =====================================

// SCORE CALCULATION

// =====================================

String getScore(unsigned long time) {

if (time < 200) {

return "EXCELLENT";

}

else if (time < 300) {

return "GOOD";

}

else if (time < 450) {

return "MEDIOCRE";

}

else if (time < 600) {

return "POOR";

}

else {

return "VERY POOR";

}

}

// =====================================

// TURN OFF ALL LEDs

// =====================================

void allLEDsOff() {

digitalWrite(SIGNAL_LED, LOW);

digitalWrite(WIN_LED, LOW);

digitalWrite(RED_LED, LOW);

digitalWrite(BAR_LED_1, LOW);

digitalWrite(BAR_LED_2, LOW);

digitalWrite(BAR_LED_3, LOW);

digitalWrite(BAR_LED_4, LOW);

digitalWrite(BAR_LED_5, LOW);

}

// =====================================

// RESULT LED

// =====================================

void showResultLED(unsigned long time) {

// Yellow GO LED OFF

digitalWrite(SIGNAL_LED, LOW);

// Green LED ON when result is displayed

digitalWrite(WIN_LED, HIGH);

// Red LED OFF

digitalWrite(RED_LED, LOW);

}

// =====================================

// PERFORMANCE BAR

// =====================================

void showPerformanceBar(unsigned long time) {

// Turn OFF all bar LEDs first

digitalWrite(BAR_LED_1, LOW);

digitalWrite(BAR_LED_2, LOW);

digitalWrite(BAR_LED_3, LOW);

digitalWrite(BAR_LED_4, LOW);

digitalWrite(BAR_LED_5, LOW);

// ===================================

// EXCELLENT: < 200 ms

// 5 LEDs

// ===================================

if (time < 200) {

digitalWrite(BAR_LED_1, HIGH);

digitalWrite(BAR_LED_2, HIGH);

digitalWrite(BAR_LED_3, HIGH);

digitalWrite(BAR_LED_4, HIGH);

digitalWrite(BAR_LED_5, HIGH);

}

// ===================================

// GOOD: 200 - 299 ms

// 4 LEDs

// ===================================

else if (time < 300) {

digitalWrite(BAR_LED_1, HIGH);

digitalWrite(BAR_LED_2, HIGH);

digitalWrite(BAR_LED_3, HIGH);

digitalWrite(BAR_LED_4, HIGH);

}

// ===================================

// MEDIOCRE: 300 - 449 ms

// 3 LEDs

// ===================================

else if (time < 450) {

digitalWrite(BAR_LED_1, HIGH);

digitalWrite(BAR_LED_2, HIGH);

digitalWrite(BAR_LED_3, HIGH);

}

// ===================================

// POOR: 450 - 599 ms

// 2 LEDs

// ===================================

else if (time < 600) {

digitalWrite(BAR_LED_1, HIGH);

digitalWrite(BAR_LED_2, HIGH);

}

// ===================================

// VERY POOR: >= 600 ms

// 1 LED

// ===================================

else {

digitalWrite(BAR_LED_1, HIGH);

}

}

// =====================================

// SHOW RESULT

// =====================================

void showResult() {

String score = getScore(reactionTime);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Time: ");

lcd.print(reactionTime);

lcd.print(" ms");

lcd.setCursor(0, 1);

lcd.print(score);

}

// =====================================

// SETUP

// =====================================

void setup() {

// Main LEDs

pinMode(SIGNAL_LED, OUTPUT);

pinMode(WIN_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

// Push button

pinMode(BUTTON_PIN, INPUT_PULLUP);

// Performance bar LEDs

pinMode(BAR_LED_1, OUTPUT);

pinMode(BAR_LED_2, OUTPUT);

pinMode(BAR_LED_3, OUTPUT);

pinMode(BAR_LED_4, OUTPUT);

pinMode(BAR_LED_5, OUTPUT);

// Turn everything OFF

allLEDsOff();

// Initialize I2C

Wire.begin();

// Initialize LCD

lcd.init();

lcd.backlight();

// Show starting screen

showReady();

}

// =====================================

// MAIN LOOP

// =====================================

void loop() {

// ===================================

// WAITING FOR BUTTON

// ===================================

if (gameState == WAITING) {

if (buttonPressed()) {

allLEDsOff();

gameState = COUNTDOWN;

// ================================

// COUNTDOWN

// ================================

showCountdown(3);

delay(1000);

showCountdown(2);

delay(1000);

showCountdown(1);

delay(1000);

// ================================

// GO

// ================================

showGo();

// Yellow LED stays ON during GO

digitalWrite(SIGNAL_LED, HIGH);

// Start reaction timer

startTime = millis();

gameState = GO;

}

}

// ===================================

// WAITING FOR REACTION

// ===================================

else if (gameState == GO) {

if (buttonPressed()) {

// Calculate reaction time

reactionTime = millis() - startTime;

// Turn OFF yellow GO LED

digitalWrite(SIGNAL_LED, LOW);

// Show result on LCD

showResult();

// Turn ON green result LED

showResultLED(reactionTime);

// Show performance bar

showPerformanceBar(reactionTime);

gameState = RESULT;

}

}

// ===================================

// RESULT DISPLAY

// ===================================

else if (gameState == RESULT) {

// Keep result displayed for 3 seconds

delay(3000);

// Turn OFF all LEDs

allLEDsOff();

// Return to ready screen

showReady();

gameState = WAITING;

}

// ===================================

// FALSE START

// ===================================

else if (gameState == FALSE_START) {

allLEDsOff();

digitalWrite(RED_LED, HIGH);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("TOO EARLY!");

lcd.setCursor(0, 1);

lcd.print("False Start!");

delay(2500);

digitalWrite(RED_LED, LOW);

showReady();

gameState = WAITING;

}

}

PROGRAM : DIAGRAM.JSON

---------------------------------------

{

"version": 1,

"author": "Reaction Speed Tester",

"editor": "wokwi",

"parts": [

{

"type": "board-st-nucleo-c031c6",

"id": "nucleo",

"top": 0,

"left": 0,

"attrs": {}

},

{

"type": "wokwi-lcd1602",

"id": "lcd",

"top": -20,

"left": 330,

"attrs": {

"pins": "i2c",

"i2cAddress": "0x27"

}

},

{

"type": "wokwi-led",

"id": "led_signal",

"top": 100,

"left": 330,

"attrs": {

"color": "yellow"

}

},

{

"type": "wokwi-led",

"id": "led_win",

"top": 100,

"left": 380,

"attrs": {

"color": "green"

}

},

{

"type": "wokwi-resistor",

"id": "r1",

"top": 140,

"left": 330,

"attrs": {

"value": "220"

}

},

{

"type": "wokwi-resistor",

"id": "r2",

"top": 140,

"left": 380,

"attrs": {

"value": "220"

}

},

{

"type": "wokwi-pushbutton",

"id": "button",

"top": 100,

"left": 450,

"attrs": {

"color": "blue",

"key": " "

}

},

{

"type": "wokwi-led",

"id": "bar_led1",

"top": 390,

"left": 20,

"attrs": {

"color": "green"

}

},

{

"type": "wokwi-led",

"id": "bar_led2",

"top": 390,

"left": 60,

"attrs": {

"color": "green"

}

},

{

"type": "wokwi-led",

"id": "bar_led3",

"top": 390,

"left": 100,

"attrs": {

"color": "yellow"

}

},

{

"type": "wokwi-led",

"id": "bar_led4",

"top": 390,

"left": 140,

"attrs": {

"color": "red"

}

},

{

"type": "wokwi-led",

"id": "bar_led5",

"top": 390,

"left": 180,

"attrs": {

"color": "red"

}

},

{

"type": "wokwi-resistor",

"id": "bar_r1",

"top": 430,

"left": 20,

"attrs": {

"value": "220"

}

},

{

"type": "wokwi-resistor",

"id": "bar_r2",

"top": 430,

"left": 60,

"attrs": {

"value": "220"

}

},

{

"type": "wokwi-resistor",

"id": "bar_r3",

"top": 430,

"left": 100,

"attrs": {

"value": "220"

}

},

{

"type": "wokwi-resistor",

"id": "bar_r4",

"top": 430,

"left": 140,

"attrs": {

"value": "220"

}

},

{

"type": "wokwi-resistor",

"id": "bar_r5",

"top": 430,

"left": 180,

"attrs": {

"value": "220"

}

}

],

"connections": [

["nucleo:D14", "lcd:SDA", "green"],

["nucleo:D15", "lcd:SCL", "blue"],

["nucleo:5V", "lcd:VCC", "red"],

["nucleo:GND.1", "lcd:GND", "black"],

["nucleo:D2", "led_signal:A", "yellow"],

["led_signal:C", "r1:1", "yellow"],

["r1:2", "nucleo:GND.2", "black"],

["nucleo:D3", "led_win:A", "green"],

["led_win:C", "r2:1", "green"],

["r2:2", "nucleo:GND.2", "black"],

["nucleo:D4", "button:1.l", "blue"],

["button:2.l", "nucleo:GND.2", "black"],

["nucleo:D5", "bar_led1:A", "green"],

["bar_led1:C", "bar_r1:1", "green"],

["bar_r1:2", "nucleo:GND.2", "black"],

["nucleo:D6", "bar_led2:A", "green"],

["bar_led2:C", "bar_r2:1", "green"],

["bar_r2:2", "nucleo:GND.2", "black"],

["nucleo:D7", "bar_led3:A", "yellow"],

["bar_led3:C", "bar_r3:1", "yellow"],

["bar_r3:2", "nucleo:GND.2", "black"],

["nucleo:D8", "bar_led4:A", "red"],

["bar_led4:C", "bar_r4:1", "red"],

["bar_r4:2", "nucleo:GND.2", "black"],

["nucleo:D9", "bar_led5:A", "red"],

["bar_led5:C", "bar_r5:1", "red"],

["bar_r5:2", "nucleo:GND.2", "black"]

]

}

LIBRARIES USED :  LIQUIDCRYSTAL I2C

Program link : https://wokwi.com/projects/478136568287310849